COBRETTIVIPERV8.COM

Updated Friday, 31 July 2026

Chevy Small Block Recommended Clearances

Piston to Bore			.005”-0055” measured at centre line of wrist pin hole. Perpendicular to pin.

Forged Pistons			Finish bores with #500 grit stones or equivalent.

Piston Ring			Minimum end clearance top= .002” second= .016” oil= .016”

Wrist Pin			.0004”-.0008” in piston. .0005”-.0007” in rod for floating pin

Rod Bearing			.002”-.0025”

Side Clearance		.010”-.020”

Main Bearing			.002”-.003” minimum preferred .005”-.007” end play

Piston to Top of Block	.015”-.020” average below deck. No part of piston except dome to be higher than deck of bolt

Deck Height	Deck height specified is for a .018” steel head gasket. If a thicker head gasket is used, piston to cylinder head clearance of .035” should be considered as minimum

Valve Lash	Follow camshaft manufacturer’s recommendations (always)

Valve to Piston	.020” exhaust and intake at 0 valve lash. Note; these are to be considered minimum clearances

Clearance	For an engine that runs below 8,000rpm. If you intend to run higher than that, more clearance should be allowed

The data herein has been verified as far as possible. We disclaim any liability incurred in connection with the use of this Data or any specific details mentioned herein.